LAB 05 – Part 05_Extended ISA

Group 01—E/20/381 Somathilaka A.N.T

E/20/385 Sriyarathna D.H

01.Instruction Set

Table 01 – mapping between instructions with their corresponding Operational code

| Instruction | Opcode |
| --- | --- |
| loadi | 0000 0000 |
| mov | 0000 0001 |
| add | 0000 0010 |
| sub | 0000 0011 |
| and | 0000 0100 |
| or | 0000 0101 |
| j | 0000 0110 |
| beq | 0000 0111 |
| bne | 0000 1000 |
| sra | 0000 1001 |
| sla | 0000 1010 |
| srl | 0000 1011 |
| sll | 0000 1100 |
| ror | 0000 1101 |
| rol | 0000 1110 |
| mult | 0000 1111 |

ALU Functions along with the ALUOP code and it’s delay.

Table 02 – ALU units with their corresponding ALUOP code and their delays

| ALU operation | ALUOP code | Delay |
| --- | --- | --- |
| forward | 000 | #1 |
| add | 001 | #2 |
| and | 010 | #1 |
| or | 011 | #1 |
| shift | 100 | #1 |
| rotation | 101 | #1 |
| multiplication | 110 | #3 |

02. Control Unit Signals

Table 03 – mapping between instruction opcodes and their decoded signals

| Instruction | opcode | write- enable | istwo- comp | isreg- out | ALUop | beq | jump | bne | islogicalorisleft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| loadi | 0000 0000 | 1 | 0 | 0 | 000 | 0 | 0 | 0 | 00 |
| mov | 0000 0001 | 1 | 0 | 1 | 000 | 0 | 0 | 0 | 00 |
| add | 0000 0010 | 1 | 0 | 1 | 001 | 0 | 0 | 0 | 00 |
| sub | 0000 0011 | 1 | 1 | 1 | 001 | 0 | 0 | 0 | 00 |
| and | 0000 0100 | 1 | 0 | 1 | 010 | 0 | 0 | 0 | 00 |
| or | 0000 0101 | 1 | 0 | 1 | 011 | 0 | 0 | 0 | 00 |
| j | 0000 0110 | 0 | 0 | 0 | 000 | 0 | 1 | 0 | 00 |
| beq | 0000 0111 | 0 | 1 | 1 | 001 | 1 | 0 | 0 | 00 |
| bne | 0000 1000 | 0 | 1 | 0 | 001 | 0 | 0 | 1 | 00 |
| sra | 0000 1001 | 1 | 0 | 0 | 100 | 0 | 0 | 0 | 00 |
| sla | 0000 1010 | 1 | 0 | 0 | 100 | 0 | 0 | 0 | 01. |
| srl | 0000 1011 | 1 | 0 | 0 | 100 | 0 | 0 | 0 | 10 |
| sll | 0000 1100 | 1 | 0 | 0 | 100 | 0 | 0 | 0 | 11 |
| ror | 0000 1101 | 1 | 0 | 0 | 101 | 0 | 0 | 0 | 00 |
| rol | 0000 1110 | 1 | 0 | 0 | 101 | 0 | 0 | 0 | 01 |
| mult | 0000 1111 | 1 | 0 | 1 | 110 | 0 | 0 | 0 | 00 |

In above,

- writeenable: Write Enable signal sent to the Register File to enable writing to a register.

- istwoscomp: Selects the twos complement from the register output to perform subtraction.

- isregout: Selects between the immediate value and the register out 2.

- ALUop: ALU Operation Code sent to the ALU.

- beq: Selects the target address to override the PC to execute a branch if equal instruction.

- jump: Select the target address to override the PC to execute a jump instruction.

- bne: Select the target address to override the PC to execute a branch if NOT equal instruction.

- islogicalorisleft: Selects between shifting right and shifting left, this is done by reversing the order of the 8-bit   value from the register output.

03.Shift Unit

Figure 1.0 – hardware design for shift Unit

All shifting functionalities were implemented in a single shift unit. Therefore the timing diagram for all shifting functionalities (arithmetic right shift, arithmetic left shift, logical right shift, logical left shift) as follows.

Figure 1.1 – timing diagram for the all shifting functionalities

Figure 1.2 – GTKwave file of shift unit functions

04.Rotate Right and Rotate Left Instructions

Figure 2.0 – Hardware implementation of the rotation unit

Due to the lack of ALUOP in the hardware design we used cascade muxes to implement both the right rotation and left rotation.

Therefore both rotations had the same time delay as shown in figure 2.1.

Figure 2.1 – timing diagram for the rotation unit

Figure 2.2 – GTKwave file of rotation unit functions

05.Multiplication Instructions

When doing multiplication we used bitwise AND, logical shifting, and full adder to obtain the desired output the arithmetic functionalities are shown below example.

Example:

0000 0101

Assume we need to multiply 3 and 5.

0000 0011

0000 0011

0000 0011

0000 0000

0000 0000

15

0000 1111

0000 0000

So we get one bit at a time of one number and do AND logic function with other numbers to all 8bits respectively. After that we left shift the result the first one by 1 and left shift the second result by 2 up to the last result respectively. After that, we add the shift results to obtain the final result.

Figure 3.1 – timing diagram of the multiplication

Figure 3.2 – GTKwave of the multiplication

06. Branch not Equal

An additional control signal was generated from the control unit (bne). If bne is “1” and “zero” is not equal to 0, then the PC will be written the target address instead of PC+4.

For an example let’s consider below assembly code

loadi 0 0x05

loadi 1 0x03

bne 0 1 0x01

loadi 2 0x01

mult 2 0 1

Figure 4.1 – GTKwave belong to above assembly code